The DjangoServidor service is pausing because waitress fails to bind to the hardcoded IP address 192.168.1.193. This is likely due to a change in the server's network configuration or the interface being down.

## Proposed Changes

### Configuration

#### [MODIFY]

#### server.py

- Change the host parameter in serve() from '192.168.1.193' to '0.0.0.0'.

- This allows the server to listen on all available network interfaces, making it robust against IP address changes.

## Verification Plan

### Automated Verification

- Manual Script Execution:

  - Run e:\AmandaBoutique\.venv\Scripts\python.exe e:\AmandaBoutique\server.py

  - Verify that it starts without the OSError: [WinError 10049].

  - Terminate the manual process (Ctrl+C).

- Service Verification:

  - Run nssm start DjangoServidor.

  - Run nssm status DjangoServidor and ensure it reports SERVICE_RUNNING.

  - Wait a few seconds and check status again to ensure it stays running.